CVSD Verilog Programming Midterm

S3 Texture Compression Engine

- Preparation

Please first start the X-window and putty (remember to enable X11 forwarding). Since Design Compiler is required in this test, please use cad27~29 or cad34~40. You can source the “/home/raid1_1/.cshrc” if your setup file is not complete. After logging into your own account, type “cp -R ~cvsd/06F/midterm/ ./” to copy all data for this test.

- Function Description

S3 texture compression method is proposed the 3D graphic chip company – S3, and it is also adopted as part of the 3D graphic standard. Today you’re going to implement a simplified version of S3 texture compression engine. Figure 1 is the block diagram.

Texture is a 4*4 array which consists of 16 texel_data, and each texel_data is an 8-bit unsigned binary value. The compression algorithm is formulated as follows. Please refer to Figure 2 for the symbol notations.

Tn = texel_type(Dn) = x where x is chosen to minimize |Dn-Lx| (x = 0~3, n = 0~15)

and where L0=max(D0,D1,D2,...,D15), L3=min(D0,D1,D2,...,D15),

L1=23⋅L0+13⋅L3, L2=13⋅L0+23⋅L3.

Obviously the texel_type is a 2-bit unsigned value. The 1/3 and 2/3 multiplications are actually another implementation issue, but in this test, please just simplify them into:

13≃14+116, 23≃12+18 example:13value[7:0]≃value[7:2]+value[7:4]

In case that one texel_data can map to two texel_type (if the two absolute differences are the same), please choose the smaller texel_type value.

- Timing Diagram

First your design should be reset when the rst signal becomes low (either synchronous or asynchronous is ok). Then the start signal will be clamped high for one cycle. One cycle after the start signal, the 16 texet_data are sequentially sent into your design. Once the input phase is over, the test bench starts to wait for your design’s output. If the valid signal is found to be clamped high at the clock falling edge, the testbench will compare the texel_type value with the correct answer. Please note that after the testbench counts all 16 texel_type have been sensed, which means this compression is over, the testbench may start the next run of compression. Design FSM carefully!

- Requirements

In this test, you have to finish the Verilog RTL code, RTL simulation, synthesis using Design Vision, and gate level simulation. The more steps you finish, the higher is your score. During the test, you may need to use the xemacs, verilog, dv, debussy, and nLint -gui to finish the design. For the synthesis, you can use any kind of timing, load, and delay setup. You just need to finish the synthesis, output the gate level Verilog & SDF file for the final gate level simulation. For gate level simulation, you need to use TSMC file “/home/raid1_1/cic/CBDK018_TSMC_Artisan/CIC/Verilog/tsmc18.v”.

- Submission Guideline

The 3 files to be submitted: s3tc_comp.v & s3tc_comp_syn.v & s3tc_comp_syn.sdf

Of course, if you only finish the RTL, just submit the first file. However, no matter you have 3 or 1 file(s), you have to tar your codes before submission. Please use the instruction “tar -cvf s3tc.tar *” to get the tar file s3tc.tar. After this, you can connect to the upload FTP by using “ftp cad23.ee.ntu.edu.tw 3000”. The ID/PASSWD are the same as what you used before (if you don’t bring it, please ask TA). After logging in, first key in “bin”, and then use “put s3tc.tar” to upload file. You may need to use “ls” & “rm” too. Please use the file names mentioned above, and don’t change Verilog port name, or your design may not be graded. Finally you must fill following blanks, and return this test paper to TA!

ID: _________ Name: _________ Synthesis Finished (Y/N): __ (Clock Period: __ns)